Campus A

Step 1:
Configuration of Multi-Layer Switch:

CLI command:

enable

configure terminal

hostname Campus-A-MLS

! CREATE VLANs

vlan 10

name Admin

vlan 20

name Faculty

vlan 30

name Students

vlan 40

name WiFi

vlan 50

name Servers

exit

! ENABLE L3 ROUTING

ip routing

! SVIs — Gateway IP for each VLAN

interface Vlan10

ip address 10.0.0.1 255.252.0.0

no shutdown

interface Vlan20

ip address 10.4.0.1 255.252.0.0

no shutdown

interface Vlan30

ip address 10.8.0.1 255.252.0.0

no shutdown

interface Vlan40

ip address 10.12.0.1 255.252.0.0

no shutdown

interface Vlan50

ip address 10.16.0.1 255.252.0.0

no shutdown

! UPLINK PORT → Campus Router (Fa0/1)

interface FastEthernet0/1

no switchport

ip address 192.168.1.2 255.255.255.252

no shutdown

! DOWNLINK PORTS → Department Switches

interface FastEthernet0/2

switchport mode access

switchport access vlan 10

no shutdown

interface FastEthernet0/3

switchport mode access

switchport access vlan 20

no shutdown

interface FastEthernet0/4

switchport mode access

switchport access vlan 30

no shutdown

interface FastEthernet0/5

switchport mode access

switchport access vlan 50

no shutdown

interface FastEthernet0/6

switchport mode access

switchport access vlan 40

no shutdown

! DHCP RELAY — forward DHCP to Server

interface Vlan10

ip helper-address 10.16.0.2

interface Vlan20

ip helper-address 10.16.0.2

interface Vlan30

ip helper-address 10.16.0.2

interface Vlan40

ip helper-address 10.16.0.2

! ACLs — Traffic Restriction Between VLANs

! --- ACL for Admin VLAN (full access) ---

ip access-list extended ADMIN_POLICY

permit ip any any

exit

! --- ACL for Faculty VLAN ---

ip access-list extended FACULTY_POLICY

deny   ip 10.4.0.0 0.3.255.255 10.0.0.0 0.3.255.255

permit ip 10.4.0.0 0.3.255.255 10.16.0.0 0.3.255.255

permit ip 10.4.0.0 0.3.255.255 10.12.0.0 0.3.255.255

permit ip any any

exit

! --- ACL for Students VLAN ---

ip access-list extended STUDENT_POLICY

deny   ip 10.8.0.0 0.3.255.255 10.0.0.0 0.3.255.255

deny   ip 10.8.0.0 0.3.255.255 10.4.0.0 0.3.255.255

permit ip 10.8.0.0 0.3.255.255 10.16.0.0 0.3.255.255

permit ip any any

exit

! --- ACL for WiFi VLAN ---

ip access-list extended WIFI_POLICY

deny   ip 10.12.0.0 0.3.255.255 10.0.0.0 0.3.255.255

deny   ip 10.12.0.0 0.3.255.255 10.4.0.0 0.3.255.255

permit ip 10.12.0.0 0.3.255.255 10.16.0.0 0.3.255.255

permit ip any any

exit

! APPLY ACLs to SVIs

interface Vlan10

no ip access-group ADMIN_POLICY in

interface Vlan20

ip access-group FACULTY_POLICY in

interface Vlan30

ip access-group STUDENT_POLICY in

interface Vlan40

ip access-group WIFI_POLICY in

end

write memory

Step 2:
Configuration of Access Switches:

CLI Commands:

Admin Switch:

enable

configure terminal

hostname Campus-A-Admin-SW

vlan 10

name Admin

exit

interface FastEthernet0/1

switchport mode access

switchport access vlan 10

no shutdown

interface range FastEthernet0/2 - 24

switchport mode access

switchport access vlan 10

no shutdown

end

write memory

Student Switch:

enable

configure terminal

hostname Campus-A-Student-SW

vlan 30

name Students

exit

interface FastEthernet0/1

switchport mode access

switchport access vlan 30

no shutdown

interface range FastEthernet0/2 - 24

switchport mode access

switchport access vlan 30

no shutdown

end

write memory

Faculty Switch:

enable

configure terminal

hostname Campus-A-Faculty-SW

vlan 20

name Faculty

exit

interface FastEthernet0/1

switchport mode access

switchport access vlan 20

no shutdown

interface range FastEthernet0/2 - 24

switchport mode access

switchport access vlan 20

no shutdown

end

write memory

Servers Switch:

enable

configure terminal

hostname Campus-A-Servers-SW

vlan 50

name Servers

exit

interface FastEthernet0/1

switchport mode access

switchport access vlan 50

no shutdown

interface range FastEthernet0/2 - 24

switchport mode access

switchport access vlan 50

no shutdown

end

write memory

Screen Shots:

Step3:
Configuration Servers:
DHCP:

Click Server → Desktop → IP Configuration

IP Address  : 10.16.0.2

Subnet Mask : 255.252.0.0

Default GW  : 10.16.0.1

Click Server → Services → DHCP

Turn Service: ON

Pool 1 — Admin

Pool Name    : Admin_Pool

Default GW   : 10.0.0.1

DNS Server   : 10.16.0.2

Start IP     : 10.0.0.2

Subnet Mask  : 255.252.0.0

Max Users    : 100

→ Click Add

Pool 2 — Faculty

Pool Name    : Faculty_Pool

Default GW   : 10.4.0.1

DNS Server   : 10.16.0.2

Start IP     : 10.4.0.2

Subnet Mask  : 255.252.0.0

Max Users    : 200

→ Click Add

Pool 3 — Students

Pool Name    : Student_Pool

Default GW   : 10.8.0.1

DNS Server   : 10.16.0.2

Start IP     : 10.8.0.2

Subnet Mask  : 255.252.0.0

Max Users    : 1000

→ Click Add

Pool 4 — WiFi

Pool Name    : WiFi_Pool

Default GW   : 10.12.0.1

DNS Server   : 10.16.0.2

Start IP     : 10.12.0.2

Subnet Mask  : 255.252.0.0

Max Users    : 50

→ Click Add

DNS Service:

Click Server → Services → DNS

DNS Service : ON

Add records:

Name : www.campusa.com       Type: A Record    Address: 10.16.0.2

Name : mail.campusa.com      Type: A Record    Address: 10.16.0.2

→ Click Add after each

Web Service:

Click Server → Services → HTTP

HTTP Service  : ON

HTTPS Service : ON

(Default webpage is already there, you can edit the HTML if you want)


Email Service:

Click Server → Services → EMAIL

SMTP Service : ON

POP3 Service : ON

Domain Name : campusa.com

→ Click Set

Add users:

Username : admin     Password : admin123  → Click +

Username : faculty   Password : fac123    → Click +

Username : student   Password : stu123    → Click +


Test Web

textCopy

Click PC → Desktop → Web Browser

Type: http://www.campusa.com  or  http://10.16.0.2

→ Should show webpage

Test DNS

textCopy

Click PC → Desktop → Command Prompt

ping www.campusa.com

→ Should resolve to 10.16.0.2

Test Email

textCopy

Click PC → Desktop → Mail

Configure:

Your Name    : Admin User

Email        : admin@campusa.com

Incoming Mail: 10.16.0.2

Outgoing Mail: 10.16.0.2

Username     : admin

Password     : admin123

→ Click Save

→ Try Send/Receive

Screenshot:

Campus B

Step 1:
Configuration of Multi-Layer Switch:

CLI command:

enable

configure terminal

hostname Campus-B-MLS

! CREATE VLANs

vlan 10

name Admin

vlan 20

name Faculty

vlan 30

name Students

vlan 40

name WiFi

vlan 50

name Servers

exit

! ENABLE L3 ROUTING

ip routing

! SVIs — Gateway IP for each VLAN

interface Vlan10

ip address 10.32.0.1 255.252.0.0

no shutdown

interface Vlan20

ip address 10.36.0.1 255.252.0.0

no shutdown

interface Vlan30

ip address 10.40.0.1 255.252.0.0

no shutdown

interface Vlan40

ip address 10.44.0.1 255.252.0.0

no shutdown

interface Vlan50

ip address 10.48.0.1 255.252.0.0

no shutdown

! UPLINK PORT → Campus B Router (Fa0/1)

interface FastEthernet0/1

no switchport

ip address 192.168.2.2 255.255.255.252

no shutdown

! DOWNLINK PORTS → Department Switches

interface FastEthernet0/2

switchport mode access

switchport access vlan 10

no shutdown

interface FastEthernet0/3

switchport mode access

switchport access vlan 20

no shutdown

interface FastEthernet0/4

switchport mode access

switchport access vlan 30

no shutdown

interface FastEthernet0/5

switchport mode access

switchport access vlan 50

no shutdown

interface FastEthernet0/6

switchport mode access

switchport access vlan 40

no shutdown

! DHCP RELAY

interface Vlan10

ip helper-address 10.48.0.2

interface Vlan20

ip helper-address 10.48.0.2

interface Vlan30

ip helper-address 10.48.0.2

interface Vlan40

ip helper-address 10.48.0.2

! ACLs

ip access-list extended ADMIN_POLICY

permit ip 10.32.0.0 0.3.255.255 any

permit ip any any

exit

ip access-list extended FACULTY_POLICY

deny   ip 10.36.0.0 0.3.255.255 10.32.0.0 0.3.255.255

permit ip 10.36.0.0 0.3.255.255 10.48.0.0 0.3.255.255

permit ip 10.36.0.0 0.3.255.255 10.44.0.0 0.3.255.255

permit ip any any

exit

ip access-list extended STUDENT_POLICY

deny   ip 10.40.0.0 0.3.255.255 10.32.0.0 0.3.255.255

deny   ip 10.40.0.0 0.3.255.255 10.36.0.0 0.3.255.255

permit ip 10.40.0.0 0.3.255.255 10.48.0.0 0.3.255.255

permit ip any any

exit

ip access-list extended WIFI_POLICY

deny   ip 10.44.0.0 0.3.255.255 10.32.0.0 0.3.255.255

deny   ip 10.44.0.0 0.3.255.255 10.36.0.0 0.3.255.255

permit ip 10.44.0.0 0.3.255.255 10.48.0.0 0.3.255.255

permit ip any any

exit

! APPLY ACLs

interface Vlan10

ip access-group ADMIN_POLICY in

interface Vlan20

ip access-group FACULTY_POLICY in

interface Vlan30

ip access-group STUDENT_POLICY in

interface Vlan40

ip access-group WIFI_POLICY in

end

write memory

Step 2:
Configuration of Access Switches:

CLI Commands:

Admin Switch:

enable

configure terminal

hostname Campus-B-Admin-SW

vlan 10

name Admin

exit

interface FastEthernet0/1

switchport mode access

switchport access vlan 10

no shutdown

interface range FastEthernet0/2 - 24

switchport mode access

switchport access vlan 10

no shutdown

end

write memory

Student Switch:

enable

configure terminal

hostname Campus-B-Student-SW

vlan 30

name Students

exit

interface FastEthernet0/1

switchport mode access

switchport access vlan 30

no shutdown

interface range FastEthernet0/2 - 24

switchport mode access

switchport access vlan 30

no shutdown

end

write memory

Faculty Switch:

enable

configure terminal

hostname Campus-B-Faculty-SW

vlan 20

name Faculty

exit

interface FastEthernet0/1

switchport mode access

switchport access vlan 20

no shutdown

interface range FastEthernet0/2 - 24

switchport mode access

switchport access vlan 20

no shutdown

end

write memory

Servers Switch:

enable

configure terminal

hostname Campus-B-Servers-SW

vlan 50

name Servers

exit

interface FastEthernet0/1

switchport mode access

switchport access vlan 50

no shutdown

interface range FastEthernet0/2 - 24

switchport mode access

switchport access vlan 50

no shutdown

end

write memory

Screenshots:

Step3:
Configuration Servers:
DHCP:
IP Address  : 10.48.0.2

Subnet Mask : 255.252.0.0

Default GW  : 10.48.0.1

| Pool Name | Default GW | Start IP | Mask | Max Users |
| --- | --- | --- | --- | --- |
| Admin_Pool | 10.32.0.1 | 10.32.0.2 | 255.252.0.0 | 100 |
| Faculty_Pool | 10.36.0.1 | 10.36.0.2 | 255.252.0.0 | 200 |
| Student_Pool | 10.40.0.1 | 10.40.0.2 | 255.252.0.0 | 1000 |
| WiFi_Pool | 10.44.0.1 | 10.44.0.2 | 255.252.0.0 | 50 |

DNS Service:

www.campusb.com   → 10.48.0.2

mail.campusb.com  → 10.48.0.2

Email Service:

SMTP Service : ON

POP3 Service : ON

Domain Name : campusb.com

→ Click Set

Add users:

Username : admin     Password : admin123  → Click +

Username : faculty   Password : faculty    → Click +

Username : student   Password : stu123    → Click +

Screenshot:

Campus C

Step 1:
Configuration of Multi-Layer Switch:

CLI command:
 enable

configure terminal

hostname Campus-C-MLS

vlan 10

name Admin

vlan 20

name Faculty

vlan 30

name Students

vlan 40

name WiFi

vlan 50

name Servers

exit

ip routing

interface Vlan10

ip address 10.64.0.1 255.252.0.0

no shutdown

interface Vlan20

ip address 10.68.0.1 255.252.0.0

no shutdown

interface Vlan30

ip address 10.72.0.1 255.252.0.0

no shutdown

interface Vlan40

ip address 10.76.0.1 255.252.0.0

no shutdown

interface Vlan50

ip address 10.80.0.1 255.252.0.0

no shutdown

interface FastEthernet0/1

no switchport

ip address 192.168.3.2 255.255.255.252

no shutdown

interface FastEthernet0/2

switchport mode access

switchport access vlan 10

no shutdown

interface FastEthernet0/3

switchport mode access

switchport access vlan 20

no shutdown

interface FastEthernet0/4

switchport mode access

switchport access vlan 30

no shutdown

interface FastEthernet0/5

switchport mode access

switchport access vlan 50

no shutdown

interface FastEthernet0/6

switchport mode access

switchport access vlan 40

no shutdown

interface Vlan10

ip helper-address 10.80.0.2

interface Vlan20

ip helper-address 10.80.0.2

interface Vlan30

ip helper-address 10.80.0.2

interface Vlan40

ip helper-address 10.80.0.2

ip access-list extended ADMIN_POLICY

permit ip 10.64.0.0 0.3.255.255 any

permit ip any any

exit

ip access-list extended FACULTY_POLICY

deny   ip 10.68.0.0 0.3.255.255 10.64.0.0 0.3.255.255

permit ip 10.68.0.0 0.3.255.255 10.80.0.0 0.3.255.255

permit ip 10.68.0.0 0.3.255.255 10.76.0.0 0.3.255.255

permit ip any any

exit

ip access-list extended STUDENT_POLICY

deny   ip 10.72.0.0 0.3.255.255 10.64.0.0 0.3.255.255

deny   ip 10.72.0.0 0.3.255.255 10.68.0.0 0.3.255.255

permit ip 10.72.0.0 0.3.255.255 10.80.0.0 0.3.255.255

permit ip any any

exit

ip access-list extended WIFI_POLICY

deny   ip 10.76.0.0 0.3.255.255 10.64.0.0 0.3.255.255

deny   ip 10.76.0.0 0.3.255.255 10.68.0.0 0.3.255.255

permit ip 10.76.0.0 0.3.255.255 10.80.0.0 0.3.255.255

permit ip any any

exit

interface Vlan10

ip access-group ADMIN_POLICY in

interface Vlan20

ip access-group FACULTY_POLICY in

interface Vlan30

ip access-group STUDENT_POLICY in

interface Vlan40

ip access-group WIFI_POLICY in

end

write memory

Step 2:
Configuration of Access Switches:

CLI Commands:

Admin Switch:

enable

configure terminal

hostname Campus-C-Admin-SW

vlan 10

name Admin

exit

interface FastEthernet0/1

switchport mode access

switchport access vlan 10

no shutdown

interface range FastEthernet0/2 - 24

switchport mode access

switchport access vlan 10

no shutdown

end

write memory

Student Switch:

enable

configure terminal

hostname Campus-C-Student-SW

vlan 30

name Students

exit

interface FastEthernet0/1

switchport mode access

switchport access vlan 30

no shutdown

interface range FastEthernet0/2 - 24

switchport mode access

switchport access vlan 30

no shutdown

end

write memory

Faculty Switch:

enable

configure terminal

hostname Campus-C-Faculty-SW

vlan 20

name Faculty

exit

interface FastEthernet0/1

switchport mode access

switchport access vlan 20

no shutdown

interface range FastEthernet0/2 - 24

switchport mode access

switchport access vlan 20

no shutdown

end

write memory

Servers Switch:

enable

configure terminal

hostname Campus-C-Servers-SW

vlan 50

name Servers

exit

interface FastEthernet0/1

switchport mode access

switchport access vlan 50

no shutdown

interface range FastEthernet0/2 - 24

switchport mode access

switchport access vlan 50

no shutdown

end

write memory

Screenshots:

Step3:
Configuration Servers:
DHCP:
IP Address  : 10.80.0.2

Subnet Mask : 255.252.0.0

Default GW  : 10.80.0.1

| Pool Name | Default GW | Start IP | Mask | Max Users |
| --- | --- | --- | --- | --- |
| Admin_Pool | 10.64.0.1 | 10.64.0.2 | 255.252.0.0 | 100 |
| Faculty_Pool | 10.68.0.1 | 10.68.0.2 | 255.252.0.0 | 200 |
| Student_Pool | 10.72.0.1 | 10.72.0.2 | 255.252.0.0 | 1000 |
| WiFi_Pool | 10.76.0.1 | 10.76.0.2 | 255.252.0.0 | 50 |

DNS Service:

DNS: www.campusc.com / mail.campusc.com → 10.80.0.2

Email Service:

SMTP Service : ON

POP3 Service : ON

Domain Name : campusc.com

→ Click Set

Add users:

Username : admin     Password : admin123  → Click +

Username : faculty   Password : fac123    → Click +

Username : student   Password : stu123    → Click +

Screenshot:

Campus D

Step 1:
Configuration of Multi-Layer Switch:

CLI command:

enable

configure terminal

hostname Campus-D-MLS

vlan 10

name Admin

vlan 20

name Faculty

vlan 30

name Students

vlan 40

name WiFi

vlan 50

name Servers

exit

ip routing

interface Vlan10

ip address 10.96.0.1 255.252.0.0

no shutdown

interface Vlan20

ip address 10.100.0.1 255.252.0.0

no shutdown

interface Vlan30

ip address 10.104.0.1 255.252.0.0

no shutdown

interface Vlan40

ip address 10.108.0.1 255.252.0.0

no shutdown

interface Vlan50

ip address 10.112.0.1 255.252.0.0

no shutdown

interface FastEthernet0/1

no switchport

ip address 192.168.4.2 255.255.255.252

no shutdown

interface FastEthernet0/2

switchport mode access

switchport access vlan 10

no shutdown

interface FastEthernet0/3

switchport mode access

switchport access vlan 20

no shutdown

interface FastEthernet0/4

switchport mode access

switchport access vlan 30

no shutdown

interface FastEthernet0/5

switchport mode access

switchport access vlan 50

no shutdown

interface FastEthernet0/6

switchport mode access

switchport access vlan 40

no shutdown

interface Vlan10

ip helper-address 10.112.0.2

interface Vlan20

ip helper-address 10.112.0.2

interface Vlan30

ip helper-address 10.112.0.2

interface Vlan40

ip helper-address 10.112.0.2

ip access-list extended ADMIN_POLICY

permit ip 10.96.0.0 0.3.255.255 any

permit ip any any

exit

ip access-list extended FACULTY_POLICY

deny   ip 10.100.0.0 0.3.255.255 10.96.0.0 0.3.255.255

permit ip 10.100.0.0 0.3.255.255 10.112.0.0 0.3.255.255

permit ip 10.100.0.0 0.3.255.255 10.108.0.0 0.3.255.255

permit ip any any

exit

ip access-list extended STUDENT_POLICY

deny   ip 10.104.0.0 0.3.255.255 10.96.0.0 0.3.255.255

deny   ip 10.104.0.0 0.3.255.255 10.100.0.0 0.3.255.255

permit ip 10.104.0.0 0.3.255.255 10.112.0.0 0.3.255.255

permit ip any any

exit

ip access-list extended WIFI_POLICY

deny   ip 10.108.0.0 0.3.255.255 10.96.0.0 0.3.255.255

deny   ip 10.108.0.0 0.3.255.255 10.100.0.0 0.3.255.255

permit ip 10.108.0.0 0.3.255.255 10.112.0.0 0.3.255.255

permit ip any any

exit

interface Vlan10

ip access-group ADMIN_POLICY in

interface Vlan20

ip access-group FACULTY_POLICY in

interface Vlan30

ip access-group STUDENT_POLICY in

interface Vlan40

ip access-group WIFI_POLICY in

end

write memory

Step 2:
Configuration of Access Switches:

CLI Commands:

Admin Switch:

enable

configure terminal

hostname Campus-D-Admin-SW

vlan 10

name Admin

exit

interface FastEthernet0/1

switchport mode access

switchport access vlan 10

no shutdown

interface range FastEthernet0/2 - 24

switchport mode access

switchport access vlan 10

no shutdown

end

write memory

Student Switch:

enable

configure terminal

hostname Campus-D-Student-SW

vlan 30

name Students

exit

interface FastEthernet0/1

switchport mode access

switchport access vlan 30

no shutdown

interface range FastEthernet0/2 - 24

switchport mode access

switchport access vlan 30

no shutdown

end

write memory

Faculty Switch:

enable

configure terminal

hostname Campus-D-Faculty-SW

vlan 20

name Faculty

exit

interface FastEthernet0/1

switchport mode access

switchport access vlan 20

no shutdown

interface range FastEthernet0/2 - 24

switchport mode access

switchport access vlan 20

no shutdown

end

write memory

Servers Switch:

enable

configure terminal

hostname Campus-D-Servers-SW

vlan 50

name Servers

exit

interface FastEthernet0/1

switchport mode access

switchport access vlan 50

no shutdown

interface range FastEthernet0/2 - 24

switchport mode access

switchport access vlan 50

no shutdown

end

write memory

Screenshots:

Step3:
Configuration Servers:
DHCP:
IP Address  : 10.112.0.2

Subnet Mask : 255.252.0.0

Default GW  : 10.112.0.1

| Pool Name | Default GW | Start IP | Mask | Max Users |
| --- | --- | --- | --- | --- |
| Admin_Pool | 10.96.0.1 | 10.96.0.2 | 255.252.0.0 | 100 |
| Faculty_Pool | 10.100.0.1 | 10.100.0.2 | 255.252.0.0 | 200 |
| Student_Pool | 10.104.0.1 | 10.104.0.2 | 255.252.0.0 | 1000 |
| WiFi_Pool | 10.108.0.1 | 10.108.0.2 | 255.252.0.0 | 50 |

DNS Service:

DNS: www.campusd.com / mail.campusd.com → 10.112.0.2

Email Service:

SMTP Service : ON

POP3 Service : ON

Domain Name : campusd.com

→ Click Set

Add users:

Username : admin     Password : admin123  → Click +

Username : faculty   Password : fac123    → Click +

Username : student   Password : stu123    → Click +

Screenshot:

Campus E

Step 1:
Configuration of Multi-Layer Switch:

CLI command:
 enable

configure terminal

hostname Campus-E-MLS

vlan 10

name Admin

vlan 20

name Faculty

vlan 30

name Students

vlan 40

name WiFi

vlan 50

name Servers

exit

ip routing

interface Vlan10

ip address 10.128.0.1 255.252.0.0

no shutdown

interface Vlan20

ip address 10.132.0.1 255.252.0.0

no shutdown

interface Vlan30

ip address 10.136.0.1 255.252.0.0

no shutdown

interface Vlan40

ip address 10.140.0.1 255.252.0.0

no shutdown

interface Vlan50

ip address 10.144.0.1 255.252.0.0

no shutdown

interface FastEthernet0/1

no switchport

ip address 192.168.5.2 255.255.255.252

no shutdown

interface FastEthernet0/2

switchport mode access

switchport access vlan 10

no shutdown

interface FastEthernet0/3

switchport mode access

switchport access vlan 20

no shutdown

interface FastEthernet0/4

switchport mode access

switchport access vlan 30

no shutdown

interface FastEthernet0/5

switchport mode access

switchport access vlan 50

no shutdown

interface FastEthernet0/6

switchport mode access

switchport access vlan 40

no shutdown

interface Vlan10

ip helper-address 10.144.0.2

interface Vlan20

ip helper-address 10.144.0.2

interface Vlan30

ip helper-address 10.144.0.2

interface Vlan40

ip helper-address 10.144.0.2

ip access-list extended ADMIN_POLICY

permit ip 10.128.0.0 0.3.255.255 any

permit ip any any

exit

ip access-list extended FACULTY_POLICY

deny   ip 10.132.0.0 0.3.255.255 10.128.0.0 0.3.255.255

permit ip 10.132.0.0 0.3.255.255 10.144.0.0 0.3.255.255

permit ip 10.132.0.0 0.3.255.255 10.140.0.0 0.3.255.255

permit ip any any

exit

ip access-list extended STUDENT_POLICY

deny   ip 10.136.0.0 0.3.255.255 10.128.0.0 0.3.255.255

deny   ip 10.136.0.0 0.3.255.255 10.132.0.0 0.3.255.255

permit ip 10.136.0.0 0.3.255.255 10.144.0.0 0.3.255.255

permit ip any any

exit

ip access-list extended WIFI_POLICY

deny   ip 10.140.0.0 0.3.255.255 10.128.0.0 0.3.255.255

deny   ip 10.140.0.0 0.3.255.255 10.132.0.0 0.3.255.255

permit ip 10.140.0.0 0.3.255.255 10.144.0.0 0.3.255.255

permit ip any any

exit

interface Vlan10

ip access-group ADMIN_POLICY in

interface Vlan20

ip access-group FACULTY_POLICY in

interface Vlan30

ip access-group STUDENT_POLICY in

interface Vlan40

ip access-group WIFI_POLICY in

end

write memory

Step 2:
Configuration of Access Switches:

CLI Commands:

Admin Switch:

enable

configure terminal

hostname Campus-E-Admin-SW

vlan 10

name Admin

exit

interface FastEthernet0/1

switchport mode access

switchport access vlan 10

no shutdown

interface range FastEthernet0/2 - 24

switchport mode access

switchport access vlan 10

no shutdown

end

write memory

Student Switch:

enable

configure terminal

hostname Campus-E-Student-SW

vlan 30

name Students

exit

interface FastEthernet0/1

switchport mode access

switchport access vlan 30

no shutdown

interface range FastEthernet0/2 - 24

switchport mode access

switchport access vlan 30

no shutdown

end

write memory

Faculty Switch:

enable

configure terminal

hostname Campus-E-Faculty-SW

vlan 20

name Faculty

exit

interface FastEthernet0/1

switchport mode access

switchport access vlan 20

no shutdown

interface range FastEthernet0/2 - 24

switchport mode access

switchport access vlan 20

no shutdown

end

write memory

Servers Switch:

enable

configure terminal

hostname Campus-E-Servers-SW

vlan 50

name Servers

exit

interface FastEthernet0/1

switchport mode access

switchport access vlan 50

no shutdown

interface range FastEthernet0/2 - 24

switchport mode access

switchport access vlan 50

no shutdown

end

write memory

Screenshots:

Step3:
Configuration Servers:

DHCP:
IP Address  : 10.144.0.2

Subnet Mask : 255.252.0.0

Default GW  : 10.144.0.1

| Pool Name | Default GW | Start IP | Mask | Max Users |
| --- | --- | --- | --- | --- |
| Admin_Pool | 10.128.0.1 | 10.128.0.2 | 255.252.0.0 | 100 |
| Faculty_Pool | 10.132.0.1 | 10.132.0.2 | 255.252.0.0 | 200 |
| Student_Pool | 10.136.0.1 | 10.136.0.2 | 255.252.0.0 | 1000 |
| WiFi_Pool | 10.140.0.1 | 10.140.0.2 | 255.252.0.0 | 50 |

DNS Service:

DNS: www.campuse.com / mail.campuse.com → 10.144.0.2

Email Service:

SMTP Service : ON

POP3 Service : ON

Domain Name : campuse.com

→ Click Set

Add users:

Username : admin     Password : admin123  → Click +

Username : faculty   Password : fac123    → Click +

Username : student   Password : stu123    → Click +

Screenshot:

Campus F

Step 1:
Configuration of Multi-Layer Switch:

CLI command:
 enable

configure terminal

hostname Campus-F-MLS

vlan 10

name Admin

vlan 20

name Faculty

vlan 30

name Students

vlan 40

name WiFi

vlan 50

name Servers

exit

ip routing

interface Vlan10

ip address 10.160.0.1 255.252.0.0

no shutdown

interface Vlan20

ip address 10.164.0.1 255.252.0.0

no shutdown

interface Vlan30

ip address 10.168.0.1 255.252.0.0

no shutdown

interface Vlan40

ip address 10.172.0.1 255.252.0.0

no shutdown

interface Vlan50

ip address 10.176.0.1 255.252.0.0

no shutdown

interface FastEthernet0/1

no switchport

ip address 192.168.6.2 255.255.255.252

no shutdown

interface FastEthernet0/2

switchport mode access

switchport access vlan 10

no shutdown

interface FastEthernet0/3

switchport mode access

switchport access vlan 20

no shutdown

interface FastEthernet0/4

switchport mode access

switchport access vlan 30

no shutdown

interface FastEthernet0/5

switchport mode access

switchport access vlan 50

no shutdown

interface FastEthernet0/6

switchport mode access

switchport access vlan 40

no shutdown

interface Vlan10

ip helper-address 10.176.0.2

interface Vlan20

ip helper-address 10.176.0.2

interface Vlan30

ip helper-address 10.176.0.2

interface Vlan40

ip helper-address 10.176.0.2

ip access-list extended ADMIN_POLICY

permit ip 10.160.0.0 0.3.255.255 any

permit ip any any

exit

ip access-list extended FACULTY_POLICY

deny   ip 10.164.0.0 0.3.255.255 10.160.0.0 0.3.255.255

permit ip 10.164.0.0 0.3.255.255 10.176.0.0 0.3.255.255

permit ip 10.164.0.0 0.3.255.255 10.172.0.0 0.3.255.255

permit ip any any

exit

ip access-list extended STUDENT_POLICY

deny   ip 10.168.0.0 0.3.255.255 10.160.0.0 0.3.255.255

deny   ip 10.168.0.0 0.3.255.255 10.164.0.0 0.3.255.255

permit ip 10.168.0.0 0.3.255.255 10.176.0.0 0.3.255.255

permit ip any any

exit

ip access-list extended WIFI_POLICY

deny   ip 10.172.0.0 0.3.255.255 10.160.0.0 0.3.255.255

deny   ip 10.172.0.0 0.3.255.255 10.164.0.0 0.3.255.255

permit ip 10.172.0.0 0.3.255.255 10.176.0.0 0.3.255.255

permit ip any any

exit

interface Vlan10

ip access-group ADMIN_POLICY in

interface Vlan20

ip access-group FACULTY_POLICY in

interface Vlan30

ip access-group STUDENT_POLICY in

interface Vlan40

ip access-group WIFI_POLICY in

end

write memory

Step 2:
Configuration of Access Switches:

CLI Commands:

Admin Switch:

enable

configure terminal

hostname Campus-F-Admin-SW

vlan 10

name Admin

exit

interface FastEthernet0/1

switchport mode access

switchport access vlan 10

no shutdown

interface range FastEthernet0/2 - 24

switchport mode access

switchport access vlan 10

no shutdown

end

write memory

Student Switch:

enable

configure terminal

hostname Campus-F-Student-SW

vlan 30

name Students

exit

interface FastEthernet0/1

switchport mode access

switchport access vlan 30

no shutdown

interface range FastEthernet0/2 - 24

switchport mode access

switchport access vlan 30

no shutdown

end

write memory

Faculty Switch:

enable

configure terminal

hostname Campus-F-Faculty-SW

vlan 20

name Faculty

exit

interface FastEthernet0/1

switchport mode access

switchport access vlan 20

no shutdown

interface range FastEthernet0/2 - 24

switchport mode access

switchport access vlan 20

no shutdown

end

write memory

Servers Switch:

enable

configure terminal

hostname Campus-F-Servers-SW

vlan 50

name Servers

exit

interface FastEthernet0/1

switchport mode access

switchport access vlan 50

no shutdown

interface range FastEthernet0/2 - 24

switchport mode access

switchport access vlan 50

no shutdown

end

write memory

Screenshots:

Step3:
Configuration Servers:
DHCP:
IP Address  : 10.176.0.2

Subnet Mask : 255.252.0.0

Default GW  : 10.176.0.1

| Pool Name | Default GW | Start IP | Mask | Max Users |
| --- | --- | --- | --- | --- |
| Admin_Pool | 10.160.0.1 | 10.160.0.2 | 255.252.0.0 | 100 |
| Faculty_Pool | 10.164.0.1 | 10.164.0.2 | 255.252.0.0 | 200 |
| Student_Pool | 10.168.0.1 | 10.168.0.2 | 255.252.0.0 | 1000 |
| WiFi_Pool | 10.172.0.1 | 10.172.0.2 | 255.252.0.0 | 50 |

DNS Service:

DNS: www.campusf.com / mail.campusf.com → 10.176.0.2

Email Service:

SMTP Service : ON

POP3 Service : ON

Domain Name : campusf.com

→ Click Set

Add users:

Username : admin     Password : admin123  → Click +

Username : faculty   Password : fac123    → Click +

Username : student   Password : stu123    → Click +

Screenshot:

Step 3:

Routing:

Cli Commands:

OSPF(MLS<->Router) and Static Routes on Each Campus Router → Central Router:

Campus A Router:

enable

conf t

ip route 0.0.0.0 0.0.0.0 10.200.0.2

router ospf 1

default-information originate

end

Campus B Router:

enable

conf t

ip route 0.0.0.0 0.0.0.0 10.200.0.5

router ospf 1

default-information originate

end

Campus C Router:

enable

conf t

ip route 0.0.0.0 0.0.0.0 10.200.0.13

router ospf 1

default-information originate

end

Campus D Router:

enable

conf t

ip route 0.0.0.0 0.0.0.0 10.200.0.17

router ospf 1

default-information originate

end

Campus E Router:

enable

conf t

ip route 0.0.0.0 0.0.0.0 10.200.0.9

router ospf 1

default-information originate

end

Campus F Router:

enable

conf t

ip route 0.0.0.0 0.0.0.0 10.200.0.22

router ospf 1

default-information originate

end

Campus A MLS:

enable

conf t

router ospf 1

router-id 1.1.1.1

network 10.0.0.0 0.255.255.255 area 0

network 192.168.1.0 0.0.0.3 area 0

end

Campus B MLS:

enable

conf t

router ospf 1

router-id 2.2.2.2

network 10.32.0.0 0.0.31.255 area 0

network 192.168.2.0 0.0.0.3 area 0

end

Campus C MLS:

enable

conf t

router ospf 1

router-id 3.3.3.3

network 10.64.0.0 0.0.31.255 area 0

network 10.200.0.4 0.0.0.3 area 0

end

Campus D MLS:

enable

conf t

router ospf 1

router-id 4.4.4.4

network 10.96.0.0 0.0.31.255 area 0

network 10.200.0.8 0.0.0.3 area 0

end

Campus E MLS:

enable

conf t

router ospf 1

router-id 5.5.5.5

network 10.128.0.0 0.0.31.255 area 0

network 10.200.0.12 0.0.0.3 area 0

end

Campus F MLS:

enable

conf t

router ospf 1

router-id 6.6.6.6

network 10.160.0.0 0.0.31.255 area 0

network 10.200.0.16 0.0.0.3 area 0

end

Configure Central Router:

enable

conf t

hostname Central-Router

! Campus A link (10.200.0.0/30)

interface Serial0/0/0

ip address 10.200.0.2 255.255.255.252

no shutdown

! Campus B link (10.200.0.4/30)

interface Serial0/0/1

ip address 10.200.0.5 255.255.255.252

no shutdown

! Campus E link (10.200.0.8/30)

interface Serial0/1/0

ip address 10.200.0.9 255.255.255.252

no shutdown

! Campus F link (10.200.0.20/30)

interface Serial0/1/1

ip address 10.200.0.22 255.255.255.252

no shutdown

! Campus C link (10.200.0.12/30)

interface Serial0/2/0

ip address 10.200.0.13 255.255.255.252

no shutdown

! Campus D link (10.200.0.16/30)

interface Serial0/3/0

ip address 10.200.0.17 255.255.255.252

no shutdown

! Static routes to each campus /11 block

ip route 10.0.0.0 255.224.0.0 10.200.0.1

ip route 10.32.0.0 255.224.0.0 10.200.0.6

ip route 10.64.0.0 255.224.0.0 10.200.0.14

ip route 10.96.0.0 255.224.0.0 10.200.0.18

ip route 10.128.0.0 255.224.0.0 10.200.0.10

ip route 10.160.0.0 255.224.0.0 10.200.0.21

end

Screenshots:

Step 4:
Nat Table on central router:
Cli Commands:

conf t

interface FastEthernet0/0

ip nat inside

interface FastEthernet0/1

ip nat inside

interface Serial0/0/0

ip nat inside

interface Serial0/0/1

ip nat inside

interface Serial0/1/0

ip nat inside

interface Serial0/1/1

ip nat inside

interface Serial0/2/0

ip nat inside

interface Serial0/3/0

ip nat inside

interface Serial0/2/1

ip nat outside

access-list 1 permit 10.0.0.0 0.255.255.255

ip nat inside source list 1 interface Serial0/2/1 overload

end

and

conf t

ip route 0.0.0.0 0.0.0.0 203.0.113.2

interface Serial0/2/1

ip address 203.0.113.1 255.255.255.252

no shutdown

end

ISP Router:

conf t

ip route 0.0.0.0 0.0.0.0 203.0.113.2

interface Serial0/2/1

ip address 203.0.113.1 255.255.255.252

no shutdown

end

Screenshots:

Step 5:
Network Security:

Cli Commands:

Screenshots: